| ITEM NOS. | #### DESCRIPTION OF EQUIPMENT/SERVICES/WORKS | QTY | UNIT PRICE (RM) | TOTAL PRICE (RM) |
| --- | --- | --- | --- | --- |
| 1.0 1.1 | PRELIMINARIES Investigating necessary existing data and information relevant to the project such as general project site layout, existing site condition and everything necessary to undertake the job. To prepare two (2) sets of as built drawing in hardcopy & one (1) set of softcopy in pen drive detailing the design works and finishes as specified by the clients. Protection of existing finishes to all areas affected and making good of defects to the damages existing structure, Clearing and cleaning to all areas before execution of the work and upon completion including transportation. Temporary Facilities & Services for works, Safe Access, Project Signboard and Hoarding Board. Provision for the Bank Guarantee as an Earnest Money, Public Liability Insurance and Workman Compensation insurance. Defects Liability Period (DLP) to all work related to project shall be one (1) year after handing over process to MIMOS. | LS |  |  |
| 2.0 | CONSTRUCTION WORKS Supply of tools, materials, labours, equipment, machineries, professional supervision and everything else required for proper execution of the works and NOT limited to the following; Note : Refer drawing for details - Attachment 1 |  |  |  |
| 2.1 | Demolishing of old building, clearing of debris and vegetation and remove from site. | LS |  |  |
| 2.2 | Manual excavation, keep for backfilling and partly remove from site. | LS |  |  |
| 2.3 | Construct Galvanized Iron (GI) fence footage of 600 mm thick x 150 mm (W) cast in-situ lean concrete BRC A7,sides formwork and concrete G25 and others to complete. | 600 m |  |  |
| 2.4 | Construct GI ‘V’ type fence post for installation of barbed wire. Size of the fence post :- - 50 mm (L) x 75 mm (W) x 2400 mm (H) - 100 mm (L) x 100 mm (W) x 3000 mm (H) | LS |  |  |
| 2.5 | Construct 150 mm thick x 300 mm (W) x 300 mm (L) cast in-situ of lean concrete, BRC A7 sides formwork and concrete G25 as footage for fence post. | LS |  |  |
| 2.6 | Supply and install 2400 mm (W) x 2400 mm (H) Galvanized Iron Fencing c/w diamond chainlink mesh , barbed wire and other required accessories. Size of mesh : 40 mm x 114 mm Size of GI : 4.5 mm x 7.5 mm | 600 m |  |  |
| 2.7 | Supply and install 2400 mm (H) x 1000 mm (W) GI single leaf door. | 2 nos |  |  |
| 2.8 | Close turf affected area to include Backfilling side of ground beam spread level. | LS |  |  |
| 2.9 | Site cleaning upon completion. | LS |  |  |
| 3.0 | Total Amount for items (1.0 to 2.9) |  |  |  |
| 4.0 | OTHER REQUIREMENTS (OPTIONAL) Other items of the Sebutharga’s obligation not in the technical specification or elsewhere. Please state and describe all items forming part of the above works:- a) b) c) d) | LS LS LS LS |  |  |
|  | Total Amount for item 4.0 |  |  |  |